DriverCentral Listing Brief

Product Name

Adaptive Lighting, Shade & Climate Automation for Control4

One-Sentence Introduction

This is an adaptive automation driver for Control4 residential projects. It can automatically control lighting, shades, and temperature based on room occupancy, time, sunrise and sunset, weather environment, manual operation, and automation style.

Product Overview

Adaptive Lighting, Shade & Climate Automation for Control4 is an advanced driver used to enhance the lighting, shade, and climate automation experience in Control4 residential projects.

Traditional Control4 automation usually relies on fixed schedules, simple motion triggering, or a large amount of manually written logic. On this basis, this driver adds a smarter decision layer. By comprehensively judging whether a room is occupied, the current time period, sunrise and sunset, weather, ambient temperature, user manual operations, and scene preferences, it automatically selects more appropriate lighting, shade, and climate control.

This driver is not intended to completely replace Control4 Composer programming, nor is it intended to replace the professional design of the dealer. Instead, it serves as a more advanced automation enhancement layer, helping the dealer use less repeated logic to provide customers with a more natural and more consistent living experience.

The first commercial version is positioned with lighting and shade automation as the core, and provides comfort climate assistance. It is not a complete HVAC management system, nor is it a security, life-safety, or emergency response system.

Core Value

The goal of this driver is to help every C4 engineer quickly achieve high-level lighting, shading, and temperature automation control that conforms to residential living behavior habits.

It can help C4 engineers:

Significantly reduce a large amount of logic programming work, lowering project delivery and later maintenance costs

Without needing advanced complex programming skills, obtain intelligent lighting, shade, and comfort climate control experiences with the effect of advanced custom programming

Main Features

Lighting Automation

Trigger lighting behavior based on room occupancy status

Support different control logic for daytime, evening, night, and late night

Support time-period judgment based on sunrise / sunset

Support weather brightness context judgment

Support different brightness strategies

Support manual light on / off protection

Support user preference learning after scene triggering

Support avoiding repeated automatic control within a short time

Shade / Blind / Shading Automation

Support daytime opening strategy

Support partial closing strategy based on orientation and sunlight

Support weather-related shade judgment

Support night closing strategy

Support bedroom night special hold strategy

Support shade recheck after wake-up

Support manual shade operation protection

Support project-level configuration and debugging for special shade types

Comfort Climate Automation

Support comfort automatic control of air conditioning and floor heating

The air conditioner can enter comfort cooling / heating status based on room occupancy, indoor temperature, weather trend, and manual operation protection

When weather trend data is missing, if the indoor temperature is high, a conservative cooling fallback mode can be enabled

When the temperature is low and there is no floor heating, the air conditioner enters heating mode

After the room becomes unoccupied, the air-conditioning setpoint is adjusted in stages to achieve energy saving; when occupied, comfort is automatically restored

Floor heating can automatically adjust the comfort temperature model based on Automation Style

Away and Home execute corresponding energy-saving adjustment and restoration

Applicable Projects

This driver is suitable for the following Control4 residential projects:

Multi-room automatic lighting control

A large number of motion sensors or occupancy sensors installed

Many dimmable lights, lighting scenes, or color-temperature-adjustable fixtures

Motorized shades, blinds, or shading devices

Customers want automation to be more natural than ordinary motion-sensor switching

The dealer wants to reduce a large amount of repeated room logic programming

Limited comfort climate assistance is needed

Lighting / shade behavior needs to be optimized according to sunrise, sunset, and weather

Not Applicable Scope / Content Not Supported in the First Version

The first commercial release is not positioned as a “whole-home AI control system,” and does not provide formal automation support for the following systems:

Fresh air

Security alarm

Intrusion detection

Garage door

Fire alarm

Smoke alarm

Carbon monoxide alarm

Water leak protection

Gas valve

Medical care

Elderly care

Emergency response automation

Any life-safety or safety-critical system

These systems may be directions for future versions, but they are not within the support scope of the first commercial release.

Safety Boundary

This driver is a comfort automation driver, not a security system, life-safety system, or emergency response system.

It should not be used as the sole basis for judgment for any safety-critical action.

The installing dealer needs to be responsible for:

Enter the accurate latitude and longitude of the project location.

Room names need to include similar expressions of the room type, such as bedroom

Light names need to include similar expressions of the lighting purpose, such as walkway light

Shade names need to include the shade type and orientation, such as southeast roller shade

Correct mapping of motion sensors / occupancy sensors

All devices are in the correct rooms

Disable any room or device that is not suitable for automatic control (remove motion sensor configuration, set HVAC Control False, remove orientation from shade names)

Recommended Installation and Usage Method

Add all devices into Composer and successfully debug them

Correctly name rooms, lights, and shades

Install the driver and complete authorization

Configure motion sensors (normal corresponds to motion sensors inside rooms with natural daylight; force corresponds to motion sensors inside rooms without natural daylight)

Configure the light-off time after motion release

Select automation mode: energy saving, standard, proactive

Associate keypad buttons with the driver’s lighting scene or shade control commands

Verify whether automation control and keypad control are normal

Both motion-triggered lighting and keypad scene lighting can have their results changed through the APP

Adjust the automation style based on customer feedback

Trial period: 30 days.

Recommended Listing Short Description

Adaptive Lighting, Shade & Climate Automation for Control4 is an adaptive comfort automation driver for Control4 residential projects. It optimizes lighting, shades, and comfort linkage of air conditioning and floor heating through room occupancy, time, sunrise and sunset, weather environment, manual operation, and Automation Style, helping the dealer greatly reduce repeated room logic programming work; even without advanced complex programming skills, it can achieve a more natural, more stable, and higher-level intelligent automation effect in standard Control4 projects.

Recommended Selling Point Summary

An advanced comfort automation driver for Control4 dealers

Significantly reduce a large amount of repeated room logic programming work, lowering project delivery and later maintenance costs

Without advanced complex programming skills, it can also obtain intelligent lighting, shade, and comfort climate experiences close to the effect of advanced custom programming

With lighting and shade automation as the core, provides comfort climate assistance

Not just a simple motion-sensor switch, but judgment based on occupancy, time, sunrise and sunset, weather, and room status

Supports different lighting logic for daytime, evening, night, and late night

Supports weather brightness to reduce unnecessary daytime lighting

Supports lighting preference learning, and results adjusted by the user through the APP can be recorded

Supports automatic shade control according to orientation, sunlight, time, and night strategy

Supports manual operation protection, reducing automatic repeated overriding of user intent

Supports Eco / Balanced / Proactive three automation styles, making it easy for the dealer to quickly adjust the level of control aggressiveness

Supports air-conditioning comfort assistance: high-temperature cooling, heating when low temperature and no floor heating, staged setpoint rollback after unoccupied, and comfort restoration after occupied

Supports high-temperature conservative cooling fallback when weather trend is missing, improving climate control stability

Supports floor heating using a built-in comfort temperature model according to Automation Style

Recommended Positioning Statement

A smarter comfort automation layer for Control4 lighting, shades, and climate comfort.

Chinese reference:

Create a more natural adaptive automation layer for Control4 lighting, shades, and comfort climate control.